Eric Schooler

Sina Asadi

CS-350

October 25th, 2026

Final Report: SysTec Thermostat

Right now, we have a working prototype running on a Raspberry Pi so it’s hard not to recommend using what we have.  However each of the alternatives have their own benefits, with Pi we can use python running over I2C, control the lights with PWM, read from GPIO buttions, drive a display, and finally reports status over UART.  With Microchip and Freescale they provide I2C, PWM, GPIO, LCD, and UART support in the same way as Raspberry Pi, however, both microcontrollers would need to be written in another language in order to meet the same speed that RaspberryPi runs on python.  Freescale can use Linux, but Microchip is bare-metal only.  Currently we have implementation in Linux so Microchip, which would require a flash each time it gets updated and needs an interpenetration layer such as MicroPython, would be the slower and more difficult to implement.  Due to this, and due to additional compatibility needs, Microchip would not suit our needs.  Freescale having Linux support and better python integration would suit our needs better than Microchip.

Raspberry Pi has built in 2.4 GHz connectivity and works over TCP/IP and TLS.  The thermostat can send statuses to SysTec’s servers over HTTPS (encrypted) with the same python libraries we are already using.  Microchip alternatives use a simpler WiFi chip that saves on memory but limits the app to that specific API.  Freescale, unfortunately, does not nativly support WiFi on most models so it would not suit our needs at all.

As stated earlier, we currently rely on Linux with python and a separate display thread.  Our current model supplies 4GB of RAM with storage offers much higher depending on the supported size of microSD card.  Microchip has significantly less space and very little RAM.  A project in its current state, especially with python which is a more demanding language.  Most of the options I see are in KB of RAM, and the same for flash storage.  Freescale has more RAM capacity and external flash capabilities in general, but Linux support is not as robust as Raspberry Pi offerings.

The Raspberry Pi meets all three requirements and would need no redesign.  It supports all needed peripherals, connects using advanced WiFi protocols, and has plenty of memory for frequent sampling from cloud systems.  Microchip is cheaper, but is significantly less performant.  Freescale is closer, but does not nativly support WiFi and lesser Linux integration may require a rewrite in C.

References

With Hannan, C. (2025, July 25). Python and embedded systems: Running python on raspberry pi, Arduino, or microcontrollers. Medium. https://medium.com/@CodeWithHannan/python-and-embedded-systems-running-python-on-raspberry-pi-arduino-or-microcontrollers-3f8d33028eec

LeBlanc-Williams, M. (2018, June 30). CircuitPython libraries on linux and raspberry pi. Adafruit Learning System. https://learn.adafruit.com/circuitpython-on-raspberrypi-linux/overview

Microchip Technologies Inc. (2025). 8-Bit PIC® and AVR® MCUs. Microchip.Com. https://www.microchip.com/en-us/products/microcontrollers/8-bit-mcus